Features of 8086

The most prominent features of a 8086 microprocessor are as follows –

It has an instruction queue, which is capable of storing six instruction bytes from the memory resulting in faster processing.

It was the first 16-bit processor having 16-bit ALU, 16-bit registers, internal data bus, and 16- bit external data bus resulting in faster processing.

It is available in 3 versions based on the frequency of operation –

o 8086 → 5MHz

o 8086-2 → 8MHz

o 8086-1 → 10 MHz

It uses two stages of pipelining, i.e. Fetch Stage and Execute Stage, which improves performance.

Fetch stage can prefetch up to 6 bytes of instructions and stores them in the queue.

Execute stage executes these instructions

Difference between 8085 and 8086 microprocessor