Unsigned Binary Division

Fig: Unsigned Binary Multiplication

Division example: 7/3

Q = 7 = 0111

M = 3 = 0011

- M = 2’s complement of 3 = 1100+1 = 1101

Here,

Remainder in A = 0001

= 1

Quotient in Q     = 0010

= 2

2’s Complement Division

- Divisor is loaded in M, dividend in A and Q register.

- Dividend must be expressed as twice n bit 2’complement number.

For example,

- in -7/3

Dividend = -7 = 1001 (n bit 2’s complement number)

Here, M = 1111 1001 (twice n bit 2’s complement number)

- in 7/3

Dividend = 7 = 0111 (n bit 2’s complement number)

Here, M = 0000 0111 (twice n bit 2’s complement number)

- Shift A, Q left 1 bit position.

- If M and A has same sign, i.e., MSBs are same

Perform A            A-M

Else    Perform A            A+M

- If the operation is successful

i.e., sign of A is same before and after operation

Set Q0              1

Else if Sign of A changes

Set Q0              0 and

Restore previous value of A

- Repeat the above steps for number of bits in Q. Remainder is in A. if the sign of the divisor and dividend are same, the quotient is in Q, If the sign are different, the correct quotient is 2’s complement of Q.

Q. Perform -7/3

Ans: Dividend = -7 = 2’s complement of (0111) = 1001

Dividend (in 2n bit) = 1111 1001

So, A = 1111 Q = 1001

Divisor = M = 3 = 0011

-M = 1101

| Count | A | Q | M | Remarks |
| --- | --- | --- | --- | --- |
| 4 | 1111 | 1001 | 0011 | Inital |
| 3 | 1111 | 0010 | 0011 | Log. Shift left, A&M -+ |
|  | 0010 | 0010 | 0011 | A A+M, A&A -+ |
|  | 1111 | 0010 | 0011 | Q0 = 0, Restore A = 1111 |
| 2 | 1110 | 0100 | 0011 | Log. Shift left, A&M -+ |
|  | 0001 | 0100 | 0011 | A A+M, A&A -+ |
|  | 1110 | 0100 | 0011 | Q0 = 0, Restore A = 1110 |
| 1 | 1100 | 1000 | 0011 | Log. Shift left, A&M -+ |
|  | 1111 | 1000 | 0011 | A A+M, A&A -- |
|  | 1111 | 1001 | 0011 | Q0 = 1 |
| 0 | 1111 | 0010 | 0011 | Log. Shift left, A&M -+ |
|  | 0010 | 0010 | 0011 | A A+M, |
|  | 1111 | 0010 | 0011 | Q0 = 0, Restore A = 1111 |

As Remainder is in A,

R = A = 1111

For 2’s complement to decimal conversion

| 1 | 1 | 1 | 1 |
| --- | --- | --- | --- |

-8		    4		        2	                  1

= -8+4+2+1

= -1

Remainder = -1

Quotient is in Q,

When sign of divisor (+3) and (-7) are different

Correct quotient = 2’s complement of value in Q

= 2’s complement of 0010

= 1101+1

= 1110

For 2’s complement to decimal conversion

| 1 | 1 | 1 | 0 |
| --- | --- | --- | --- |

-8		    4		        2	                  1

= -8+4+2+0

= -2